Document RA-1200-00301455

#### AIRCRAFT MAINTENANCE

#### REGULATIONS

### ANG-01

WITH ROTAX 918 EP ENGINE

AIRFRAME, PROPULSION SYSTEM, AVIONICS AND RADIO-ELECTRONIC EQUIPMENT

Operational, periodic, special forms of maintenance, storage maintenance

| Information about the |  |
| --- | --- |
| Type | airplane |
| Serial number |  |
| Registration number |  |
| Affiliation | ANG Patriot-Ukraine Ltd. |
| Start of |  |

Reprinting and duplication  of the Maintenance Regulations for ANG-01 airplane with ROTAX 918 EP engine is prohibited.

Each copy of the Regulations is valid only for the ANG-01 airplane whose flight and registration number is indicated on the front cover.

Brovary June 12, 2022

Approval of document RA-1200-00301455

Development Team

AIRCRAFT MAINTENANCE REGULATIONS

### ANG-01

WITH ROTAX 918 EP ENGINE

AIRFRAME, PROPULSION SYSTEM, AVIONICS AND RADIO-ELECTRONIC EQUIPMENT

| № | Composition | Position | Compiled, reviewed, approved | Caption |
| --- | --- | --- | --- | --- |
| 1 |  | Head of the Aviation Technical Base | Compiled the Regulations on Maintenance in its entirety |  |
| 2 |  | Head of Powertrain Division | Inspected sections: power plant, fire protection equipment |  |
| 3 |  | Leading engineer - designer | Checked sections: airframe, landing gear, cabin, household equipment, ventilation and heating systems |  |
| 4 |  | Head of the A&R Department | Checked the sections: electrical, radio, electronic, piloting, navigation and instrumentation equipment |  |
| 5 |  | Project Manager | Checked the Maintenance Regulations in their entirety |  |
| 6 |  |  | Approved the Rules of Procedure in full |  |

# 1. CONTENTS

| No. of Section |  | Str |
| --- | --- | --- |
|  | Approval of Maintenance Regulations for ANG-01 airplane with ROTAX 918 EP engine | # 1 |
| # 1 | Table of Contents | # 2 |
| # 2 | Change registration sheet | # 4 |
| # 3 | General Provisions | # 5 |
| # 4 | Structure of the Regulations | # 6 |
| # 4.1 | Operational maintenance | # 6 |
| # 4.2 | Periodic maintenance | # 8 |
| # 4.3 | Maintenance during storage | # 10 |
| # 4.4 | Specialized maintenance | # 11 |
| # 5 | Technical requirements for performing maintenance work | # 12 |
| # 6 | Safety measures | # 14 |
| # 7 | Abbreviations and symbols | # 15 |
| # 8 | Operational maintenance | # 16 |
| # 8.1 | Auxiliary works | # 16 |
| # 8.1.1 | Ensuring the departure of AIRCRAFT DEPARTURE aircraft (AD | # 16 |
| # 8.1.2 | AIRCRAFT MEETING (AM) AIRCRAFT MEETING (AM) | # 17 |
| # 8.1.3 | PARKING (P) aircraft parking provision | # 18 |
| # 8.2 | Preliminary preparation - DAILY CHECK (DY) | # 19 |
| # 8.3 | Pre-flight training a - PRE-FLIGHT INSPECTION (PFI) | # 22 |
| # 8.4 | After Flight Service - AFTER FLIGHT CHECK (AFC) | # 23 |
| # 9 | Periodic maintenance | # 24 |
| # 9.1 | Periodic forms of maintenance - 100 h, 200 h, 600 h | # 24 |
| # 10 | Maintenance during storage | # 34 |
| # 10.1 | Aircraft and engine preservation works for up to 1 year | # 34 |
| # 10.2 | Work on airplane and engine after 3 months of | # 36 |
| # 10.3 | Aircraft and engine de-conditioning work | # 37 |

| # 11 | Special maintenance | # 38 |
| --- | --- | --- |
| # 11.1 | Work performed after an aircraft enters areas of intense turbulence and thunderstorms | # 38 |
| # 11.2 | Work performed after an airplane flight with exceeded operational overload | # 38 |
| # 11.3 | Work performed after a rough landing of an airplane | # 38 |
| # 11.4 | Work performed after an aircraft enters storm conditions on the ground | # 38 |
| # 12 | Lubrication regulations | # 39 |
| # 13 | Lubrication cards | # 40 |
| # 14 | Appendices | # 41 |
| # Annex 1 | Technical parameters of aircraft and engine systems monitored during maintenance work | # 41 |
| # Annex 2 | List of operations after which a check flight (overflight) of the airplane is performed | # 42 |
| # Annex 3 | Recommended operating fluids for aircraft applications | # 43 |

2. CHANGE REGISTRATION SHEET

| No. of revision | Date of change | Deleted pages | Inserted pages | Name change | Caption |
| --- | --- | --- | --- | --- | --- |

3. GENERAL PROVISIONS

3.1 Maintenance Regulations (hereinafter referred to as MRO) is the main document defining the composition (types), scope and frequency of scheduled maintenance (hereinafter referred to as MRO) of the ANG-01 airplane, ROTAX 918 EP engine, A&R equipment and their components required to maintain airworthiness of the aircraft.

3.2 The works stipulated by the RO and additional works are performed in accordance with the Airworthiness Directives (AD), flow charts (TC), technical operation instructions, as well as Service Bulletins (SB), Service Letters (SL) and Service Instructions (SI) of the manufacturers of purchased (finished) products.

3.3 Maintenance of purchased (off-the-shelf) products should be performed in accordance with the instructions of the manufacturer of these products.

3.4 Maintenance shall be performed by the owner and/or engineering staff authorized to maintain the aircraft type and responsible for the completeness and quality of their work.

3.5 When performing maintenance work, comply with occupational health and safety rules.

3.6 Technical documentation is drawn up for each performed form of maintenance and additional work.

3.7 The Regulations do not include fulfillment:

- additional Airworthiness Directives (AD), which are issued by the Competent Authority of the country of registration of the ANG-01 airplane.

- additional Service Bulletins (SB) which are issued by the Developers of the engine, propeller, DYNON SW1100 system, rescue system (these SBs should be independently tracked on the Developers' websites).

Note

Engine Developer http://www.flyrotax.com;

Air propeller developer https://angpatriotua.com/

Developer of the DYNON SW1100 system http://www.dynonavionics.com;

Developer of the rescue system http://www.galaxysky.cz.

4. REGULATION STRUCTURE

Types of maintenance of the airplane stipulated by the Regulations:

Operational Maintenance.

Periodic Maintenance.

Storage Maintenance.

# Special types of maintenance.

4.1 Operational maintenance

4.1.1 Forms of operational

4.1.1.1 Auxiliary works

AIRCRAFT DEPARTURE (AD) aircraft departure support works

Performed immediately prior to each flight.

AIRCRAFT MEETING (AM) aircraft rendezvous operations

are being fulfilled:

- immediately after landing (except for training flights);

- when refueling during training flights.

PARKING (P) Aircraft parking operations

are being fulfilled:

- after the end of the flying shift.

- if the duration of the layover before the next flight exceeds 2 hours.

4.1.1.2 Main works

Preliminary preparation - DAILY CHECK (DY)

is performed once per flight shift, during continuous flight operations.

Pre-flight training - PRE-FLIGHT INSPECTION (PFI)

is performed immediately before each flight.

After Flight Service - AFTER FLIGHT CHECK (AFC)

is performed after the airplane has landed.

Note

Once Preflight Preparation has been performed, Preflight Preparation may not be performed before the aircraft departs.

Preparation of the airplane for flight is ensured by sequential performance of auxiliary and main operations of operational maintenance.

For example:

Types of work per 1 flight of the airplane:

- DAILY CHECK (DY);

- AIRCRAFT DEPARTURE (AD) departure support work;

- AIRCRAFT MEETING (AM);

- AFTER FLIGHT CHECK (AFC);

- PARKING (P) parking enforcement activities.

4.2 Periodic maintenance

4.2.1 Periodic maintenance  scheduled at intervals of the aircraft flight time and is performed:

- after the first 25 hours of flight time;

- after 100 hours of flight time;

- after 200 hours of flight time;

- after 300 hours of flight time;

- after 400 hours of flying time;

- after 500 hours of flight time;

- after 600 hours of flight time;

- at annual maintenance (annual inspection) - ANNUAL INSPECTION (AI)

4.2.2 The periodicity of the forms shall be uniform for the aircraft, engine and avionics products.

4.2.3 For new airplanes, Periodic Maintenance is scheduled after the first 25 hours of flight time, in the amount of 100 hours.

4.2.4 Periodic forms of maintenance consist of works performed with the periodicity of 100±10 hours and additional works, the necessity of which is determined by the flight time of the airplane every 100; 200; 600 hours.

4.2.5 The performance of works corresponding to a certain operating time is allowed with a deviation of   ±10 hours, this tolerance may not be exceeded, i.e. it is allowed to perform 100-hour works with 110 hours of flight time, but the next 200-hour works must be performed with flight time of 200±10, not 210±10 hours. If maintenance is performed before the set interval, the next maintenance must be performed at the set interval (e.g., if maintenance was performed at 87 hours of flight time in the first 100 hours, the next maintenance must be performed at 187 hours of flight time).

4.2.6 A full cycle of periodic maintenance forms is 600 hours of operation, after which the cycle of maintenance forms is repeated.

4.2.7 Annual maintenance (ANNUAL INSPECTION) shall be performed every 12±1 months of aircraft operation in the amount of 100 hours.

4.2.8 The engine hours of operation and flight time for each subsequent form of Periodic Maintenance shall be recorded from multiples of the basic frequency of 100±10 hours.

# 4.2.9. Structural diagram of periodic forms of maintenance:

| Forms of maintenance | Airplane flight hours |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 25 ч | 100 ч | 200 ч | 300 ч | 400 ч | 500 ч | 600 ч | 700 ч |
| 100 ч | Х | Х | Х | Х | Х | Х | Х | Х |
| 200 ч |  |  | Х |  | Х |  | Х |  |
| 600 ч |  |  |  |  |  |  | Х |  |

Note

In the column "Airplane flight hours" the sign "X" indicates the performance of work according to the corresponding form of maintenance.

The engine run time should be totaled by summing the engine run time on the ground and in the air.

4.2.10. It is allowed to perform periodic maintenance works by stage-by-stage maintenance method.

4.3 Maintenance during storage

4.3.1 Maintenance during aircraft storage is performed during interruptions in aircraft flights and consists of:

- work to prepare the airplane and engine for storage (preservation);

- work performed on the airplane and engine during storage; - work to prepare the airplane and engine for flight after storage (de-preservation).

The maximum period of aircraft storage is 1 year, after this time it is necessary to represerve the aircraft and engine for one year.

Note

Aircraft storage time is taken into account when calculating the time of aircraft operation against the calendar life.

4.3.2 Preservation and storage shall be performed in accordance with the instructions of the aircraft and engine operating documents.

4.3.3 All routine work on the airplane and engine must be completed by the beginning of storage.

4.3.4 If the aircraft cannot be put into service after 3 months of storage, work is performed on the aircraft after 3 months of storage.

4.3.5 The airplane and engine shall be stored:

- covered;

- in good working order;

- staffed with regular equipment, tools and ground support facilities;

- with filled tanks of fuel, oil, hydraulic, brake and cooling systems.

4.3.6 The storage area (aircraft parking lot) shall be illuminated and equipped with fire extinguishing equipment.

4.3.7 In the course of aircraft storage, it is necessary to check daily: serviceability of covers and their fastening, charging of wheel tires, presence of thrust blocks under wheels and aircraft grounding.

4.3.8. After the work on preparation for storage has been completed, the aircraft shall be sealed and placed under guard.

4.3.9 After storing the airplane for more than 6 months, it is necessary to perform a control flight (overflight) of the airplane.

4.4 Special maintenance

4.4.1 Special aircraft maintenance shall be performed:                                                                                                                                                                                                             - after entering areas of intense turbulence and thunderstorm activity;

- after a flight with exceeded operational overload;                                                                                                                                               - after a rough landing;

- after hitting stormy conditions on the ground (gale force winds with snow, dust).

In other special cases, the scope and content of maintenance work is determined by a decision of the Aircraft Developer.

# After any of the above cases, the aircraft is subjected to a set of inspections and standard operations, stipulated by the current maintenance documentation, to check the condition of elements of certain areas of the aircraft structure and make a decision on the possibility of its further operation.

4.4.2 Special types of maintenance are performed regardless of the regular form of maintenance.

4.4.3 The scope of work may be changed based on the specifics of the accident and the technical condition of the airplane, by decision of the Aircraft Developer who determines the condition of the airplane after each accident.

4.4.4 All works on special type of maintenance are performed in accordance with technological maps.

Additional work not specified in the TC shall be performed in accordance with the technological guidelines prepared by the Developer.

4.4.5 A work order card is issued for works and additional works on special types of maintenance.

4.4.6 The performed types of special maintenance shall be recorded in the aircraft form.

5. TECHNICAL REQUIREMENTS FOR PERFORMING ROUTINE MAINTENANCE WORK

5.1 Timely and quality performance of maintenance ensures maintenance of the specified level of reliability and performance of the airplane

5.2 Failures and malfunctions detected by the crew in flight and detected during aircraft maintenance shall be eliminated during maintenance

5.3 During Periodic Maintenance it is necessary to check the operating time of the engine, oil, fluids and aggregates having a limited life (service life) less than the aircraft life in order to determine the need for their replacement

5.4 When replacing the engine on the aircraft, the following must be performed:

- the form of maintenance required by hours of flight time of the airplane; - works related directly to engine replacement; - works on inspection of airframe structural elements and communication areas, access to which is possible only with the engine removed.

For subsequent forms of maintenance, the replaced engine shall be maintained in the same manner as the airplane.

5.5 After performing adjustment of units or systems, it is necessary to check their operability at maximum values of parameters and compliance of the position of the controlled element with the position of the command authority.

Note

The controls are checked from the seats of both pilots.

5.6 After installation of lines and units of fuel, oil, brake, hydraulic and cooling systems, check by external inspection that connections are tight after applying operating pressure.

5.7 The sequence of RO items in the sections does not imply the same sequence of work.

Note

In the regulation column "RO Item", the numbers indicate the number of the regulation item.

The column of the regulations "Name of maintenance object and content of work" contains the list of work to be performed during aircraft maintenance.

The "Period" column of the regulation specifies at what operating time of the aviation equipment it is necessary to perform works of this item of the regulation.

The "TC" column of the regulation specifies the number of the flow chart or a reference to the document, which sets forth the methodology for performing the work of this item of the regulation. The flow charts are placed in the appropriate sections of the "ANG-01 Airplane Technical Operation Manual with ROTAX 918 EP engine".

The "Note" column of the regulation may provide any supporting information and explanations for that paragraph of the regulation.

6. SAFETY MEASURES

6.1 Persons who have studied and passed a knowledge test on occupational health and safety rules are allowed to work on the airplane.

6.2 When performing work on the airplane, it is necessary to comply with the labor safety rules set forth in the current technical operation manuals for the ANG-01 airplane, ROTAX 918 EP engine.

6.3 Before starting aircraft maintenance, take measures to prevent accidental engine startup and activation of A&R units.

To do this, check in the cab that "MASTER SWITCH" (Main switch) and "IGN A" "IGN B" (ignition circuit switches) are in the "OFF" position.

6.4 Before starting maintenance, make sure that:

- there are serviceable fire extinguishing equipment at the aircraft parking lot;

- thrust pads are installed under the wheels of the main chassis supports.

6.5 When the engine  running, it is forbidden to perform work not related to the adjustment of engine systems and units, checking its operation modes.

It is forbidden for the person performing the engine test to leave the cockpit of the airplane with the engine running.

6.6 Ground the airplane when parked for long periods of time.

6.7 When parked outside the hangar, it is prohibited to elevator the aircraft on hoists when the wind strength exceeds 7 meters per second.

7. ABBREVIATIONS AND SYMBOLS

Regulations abbreviations:

- AREO - aviation and radio electronic equipment;

- VPP - variable pitch propeller;

- OLP - spring and summer operational period;

- AC - aircraft;

- ITS - engineering and technical staff;

- OZP - autumn-winter operational period;

- LDPE - air pressure receiver;

- RV - elevator;

- POH - Flight Operation Manual (Pilot Operation Handbook)

- RDR rudder;

- RO – technical maintenance regulations;

- AMM - Technical Operation Manual (Aircraft Maintenance Manual);

- THR throttle (engine control lever);

- CS - airplane control stick;

- TC-technological chart;

- TO  technical maintenance;

- TU - technical specifications;

- AFC - After Flight Check;

- AI - Annual Inspection;

- CM - maintenance by calendar terms;

- DY – Daily check/Preliminary Preparation;

- PFI - Pre-Flight Inspection.

8. OPERATING TO.

8.1. Auxiliary works

8.1.1 Ensuring AIRCRAFT DEPARTURE (AD) aircraft departure

| item RO | Name of service object and content of works | № TC | Note |
| --- | --- | --- | --- |
| 001 | Take off the covers. | 10- 10.02 |  |
| 002 | Remove snow, ice and frost from the surface of the airplane, if any. |  |  |
| 003 | Unlock cabin and trunk door locks and open them. |  |  |
| 004 | Inspect the household equipment and cabin. |  |  |
| 005 | Remove the mooring tackle. | 10- 20.01 |  |
| 006 | Disconnect and remove the ground wire. |  |  |
| 007 | Inspect the luggage compartment. | 25-50.01 |  |
| 008 | Check the airplane's load. | 14-20.01 |  |
| 009 | Unlock rudder and ailerons. |  |  |
| 010 | Hand the plane over to the pilot. | 10-40.01 |  |
| 011 | Establish contact with the pilot. | 10-40.02 | If AD performs ITS |
| 012 | Remove the thrust pads. | 10- 10.01 |  |
| 013 | Ensure the engine is started by the pilot. | 10-40.03 | If AD performs ITS |
| 014 | Ensure the airplane is taxiing. | 10-40.03 | If AD performs ITS |

8.1.2 AIRCRAFT MEETING (AM) aircraft meeting

| item RO | Name of service object and content of works | № TC | Note |
| --- | --- | --- | --- |
| 015 | Prepare the parking lot to receive the aircraft. | 10- 10.03 | If AM performs ITS |
| 016 | Take the airplane to the parking lot | 10- 10.03 | If AM performs ITS |
| 017 | Listen to the operation of the engine | 10- 10.03 | If AM performs ITS |
| 018 | Check that the wheels of the undercarriage supports are not overheated. |  |  |
| 019 | Install the thrust pads | 10- 10.01 |  |
| 021 | Ground the airplane. |  |  |
| 022 | Inspect the household equipment and cabin | 25-10.01 | If AM performs ITS |
| 023 | Perform an airplane tow. | 09-10.01 |  |
| 024 | Obtain information from the pilot about the operation of the material part in flight. | 10- 10.03 | If AM performs ITS |
| 025 | Perform an external inspection of the airplane en-route. |  |  |
| 026 | Check the amount of fuel in the fuel tanks. |  | If AM performs ITS |
| 027 | Take over the airplane from the pilot. |  | If AM performs ITS |

8.1.3 PARKING (P) aircraft parking provision

| item RO | Name of service object and content of works | № TC | Note |
| --- | --- | --- | --- |
| 028 | Install the thrust pads | 10- 10.01 |  |
| 030 | Ground the airplane. |  |  |
| 031 | Inspect the household equipment and cabin. | 25-10.01 |  |
| 032 | Lock rudder and ailerons. |  |  |
| 033 | Docking the plane. | 10- 20.01 | If the parking lot is outside the hangar. |
| 034 | Remove the batteries from the airplane and put them in storage. |  | When the outside air temperature is below - 20°C. |
| 035 | Close and lock the cab and luggage compartment doors. |  |  |
| 036 | Seal the airplane (if necessary). |  |  |
| 037 | Cover the airplane. | 10- 10.02 |  |

8.2 Preliminary preparation - DAILY CHECK (DY)

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
|  | PRELIMINARY WORKS |  |  |
| 038 | Take the covers off the airplane. | 10-10.02 | If it was covered |
| 039 | Remove, snow, ice, frost from the surface of the airplane. | 12-30.01 | In the presence of |
| 040 | Unlock cabin and trunk door locks and open them. |  | If they were closed |
|  | PLANNER |  |  |
| 041 | Perform an external inspection of the airplane en-route. |  |  |
| 042 | Check the amount of fuel. Refuel the aircraft or drain fuel the aircraft (if necessary). | 12-10.01 12-10.02 |  |
| 043 | Drain and check the fuel sludge from the fuel tank. | 12-10.03 |  |
|  | POWER PLANT |  |  |
| 044 | Inspect the air screw. |  |  |
| 045 | Remove engine hoods, check for oil, fuel, coolant and hydraulic fluid leaks. |  |  |
| 046 | Inspect the propulsion system: engine, units and piping. | Service Manual ROTAX 918 EP_05-20-00-page 4 Operating Instructions ROTAX 918 EP_p.3.2_page. 3-5 |  |
| 047 | Check the coolant level. | Operating Instructions ROTAX 918 EP_p.3.1_page. 3-2 |  |
| 048 | Check the oil level. | Operating Instructions ROTAX 918 EP_p.3.2_page. 3-5 |  |
| 049 | Install, lock the engine hoods with the standard locks. |  |  |
| 050 | Prepare the engine for starting. Start and test the engine. | Operating Instructions ROTAX 918 EP_p.3.3; 3-4; 3-5; 3-6_str. 3-6 |  |
|  | AIRPLANE AND ENGINE CONTROL |  |  |
| 051 | Inspect (in areas accessible for inspection) and check the operation of the aircraft and engine control systems. |  |  |

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
|  | HYDRAULIC SYSTEM |  |  |
| 052 | Inspect (in places accessible for inspection) hydraulic system units and lines. |  |  |
| 053 | Check the hydraulic fluid level. |  |  |
|  | CLOSES |  |  |
| 054 | Inspect (in places accessible for inspection) and check the operation of the flap control system. Perform control release and retraction of the flaps. |  |  |
|  | CHASSI |  |  |
| 055 | Inspect the front and main landing gear supports. |  |  |
|  | HOUSEHOLD EQUIPMENT |  |  |
| 056 | Inspect the cockpit of the airplane. | 25-10.01 |  |
| 057 | Inspect pilot seats and passenger seats. Inspect pilots' and passengers' harnesses and check the operation of the harness locks. | 25-10.01 |  |
| 058 | Inspect the luggage compartments. | 25-50.01 |  |
| 059 | Check the airplane's load. |  | When cargo is available. |
| 060 | Check the function of the ventilation system with the engine running. | 21-00.01 |  |
|  | ELECTRICAL EQUIPMENT |  |  |
| 061 | Install batteries on the airplane. | 24-30.02 | If there have been withdrawals |
| 062 | Inspect aeronautical and flashing lights, landing lights. |  |  |
| 063 | Inspect electrical equipment and wiring on the airframe and engine (in places accessible for inspection). | Operating Instructions ROTAX 918 EP_p.3.1_page. 3-4 |  |
| 064 | Check the function of the electrical system. |  |  |
|  | ELECTRONIC EQUIPMENT |  |  |
| 065 | Inspect antenna devices. |  |  |
| 066 | Inspect radio electronic equipment, check radio operation. | 23-10 | DYNON AVIONICS SkyView System Installation Guide |
| 067 | Inspect aviation headsets, check their operation. | 23-50.01 | INTERCOM 403MS Installation and Users Guide |

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
|  | INSTRUMENTS AND EQUIPMENT |  |  |
| 068 | Inspect the Pitot tube. | SkyView_System_Installation_Guide-Rev_AL_v16 |  |
| 069 | Check the function of the heating element of the Pitot tube. |  |  |
| 070 | Inspect the instrumentation units in the airplane compartments and on the engine. | Instructions ROTAX 918 EP_p.3.1_page. 3-4 |  |
| 071 | Inspect the dashboard, control panels, warning lights, instruments and equipment installed on them. |  |  |
| 072 | Set the backup altimeter arrows to zero and check that the barometric pressure scale readings correspond to the actual atmospheric pressure. |  |  |

8.3 Pre-flight training - PRE-FLIGHT INSPECTION (PFI)

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
|  | PLANNER |  |  |
| 073 | Perform an external inspection of the airplane en-route. |  |  |
| 074 | Check the amount of fuel. Refuel the aircraft or drain fuel from the aircraft. | 12-10.01 12-10.02 | If necessary |
| 075 | Drain and check fuel sludge from the fuel tank and fuel filter. | 12-10.03 |  |
|  | POWER PLANT |  |  |
| 076 | Inspect the air screw. |  |  |
|  | CHASSI |  |  |
| 077 | Inspect the front and main landing gear supports. |  |  |
|  | ELECTRICAL EQUIPMENT |  |  |
| 078 | Inspect aeronautical and flashing lights, landing lights. |  |  |
|  | ELECTRONIC EQUIPMENT |  |  |
| 079 | Inspect antenna devices. |  |  |
|  | INSTRUMENTS AND EQUIPMENT |  |  |
| 080 | Inspect the Pitot tube. | SkyView_System_Installation_Guide-Rev_AL_v16 |  |
| 081 | Inspect the dashboard, control panels, warning lights, instruments and equipment installed on them. |  |  |
| 082 | Set the backup altimeter arrows to zero and check that the barometric pressure scale readings correspond to the actual atmospheric pressure. |  |  |

8.4 Post-flight maintenance - AFTER FLIGHT CHECK (AFC)

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
|  | PLANNER |  |  |
| 083 | Check that the brakes on the wheels of the chassis supports are not overheated. |  |  |
| 084 | Perform an external inspection of the airplane en-route. |  |  |
| 085 | Check the amount of fuel. Refuel the aircraft or drain fuel from the aircraft. | 12-10.01 12-10.02 | If necessary |
| 086 | Drain and check the fuel sludge from the fuel tank. |  | If necessary |
|  | POWER PLANT |  |  |
| 087 | Inspect the air screw. |  |  |
|  | CHASSI |  |  |
| 088 | Inspect the front and main landing gear supports. |  |  |
|  | ELECTRICAL EQUIPMENT |  |  |
| 089 | Inspect aeronautical and flashing lights, landing lights. |  |  |
|  | ELECTRONIC EQUIPMENT |  |  |
| 090 | Inspect antenna devices. |  |  |
|  | INSTRUMENTS AND EQUIPMENT |  |  |
| 091 | Inspect the Pitot tube. |  | SkyView_System_Installation_Guide-Rev_AL_v16 |
| 092 | Inspect the dashboard, control panels, warning lights, instruments and equipment installed on them. |  |  |

9. PERIODIC CURRENT

9.1 Periodic forms of maintenance - 100 h, 200 h, 600 h

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
|  | PRELIMINARY WORKS |  |  | If it was covered. |
| 093 | Take the covers off the airplane. | - | 10-10.02 |  |
| 094 | Remove water, snow, ice, frost from the aircraft surface (if any). | - | 12-30.01 |  |
| 095 | Perform an external inspection of the airplane en-route. | - |  |  |
| 096 | Check cleanliness of outlet openings of drain and sufﬂy pipes of fuel, oil, hydraulic systems, cooling system. | - |  |  |
| 097 | Unlock cabin and trunk door locks and open them. | - |  |  |
| 098 | Make sure that in the cabin "MASTER SWITCH" (Main switch) and "IGN A" "IGN B" (ignition circuit switches) are in "OFF" position, the safety pin of the rescue system handle is in place. | - |  |  |
| 099 | Remove, inspect, clean contaminants from engine hood covers | - | 12-20.03 |  |
| 100 | Clean and wash the airframe (inside and outside), glazing, landing gear, engine, propeller, oil and water radiator honeycomb. | - | 12- 20.03 | Service Manual ROTAX 918 EP_ 12-20-00- page 3 |
|  | PLANNER |  |  |  |
| 101 | Removable internal and external hatches, tail fairing. Remove and inspect internal and external hatches, tail fairing of the airframe. | 100 |  |  |
| 102 | Luggage compartments, doors and luggage compartment covers Inspect the luggage compartment. Inspect luggage compartment doors, gas shock absorber Check the operation of the door lock and gas damper. Inspect the small trunk on the dashboard and the luggage compartment in the armrest, their lids, check the operation of the lid locks. | 100 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
| 103 | Cab doors. Inspect cab doors and their hinge assemblies. Inspect and check operation of: door locks, door locks in closed position, gas shock absorbers. | 100 |  |  |
| 104 | Emergency "reset" of the doors. Check the emergency "reset" of the doors. | 600 |  |  |
| 105 | Fuselage. Inspect the fuselage. | 100 |  |  |
| 106 | Stabilizer. Inspect the stabilizer. | 100 |  |  |
| 107 | Altitude rudder. Inspect the elevation rudder. | 100 |  |  |
| 108 | Altitude rudder trimmer. Inspect the elevation trimmer | 100 |  |  |
| 109 | Directional rudder, directional rudder trimmer plate. Inspect rudder, rudder trimmer plate. | 100 |  |  |
| 110 | Glazing. Inspect the glazing of the lantern, fuselage windows, wing hatch covers, cockpit doors, headlight and BANO safety glass. | 100 |  |  |
| 111 | Wing. Inspect the wing. | 100 |  |  |
| 112 | Wing-to-centerfold joints Inspect the wing-to-centerfold assemblies. | 100 |  |  |
| 113 | Flaps. Inspect the flaps. | 100 |  |  |
| 114 | Ailerons, aileron trimmer plate. Inspect ailerons, aileron trimmer plate. | 100 |  |  |
| 115 | Signs, stickers, lettering. Check the presence and condition of decals, stickers and lettering inside and outside the aircraft. | 100 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
|  | CABIN |  |  |  |
| 116 | Cladding. Inspect the cabin trim from the inside. | 100 |  |  |
| 117 | Pilot seats. Remove and inspect seats, drives of seat movement mechanisms Set up the chairs. Check the ease of movement of seats, reliability of their fixation in intermediate positions | 100 |  |  |
| 118 | Armrest. Inspect the armrest. Check the opening and closing of the armrest for access to the crane and emergency landing gear release cylinders | 100 |  |  |
| 119 | Passenger Seats. Remove and inspect the passenger seat cushions. Install the cushions and check that they are securely fastened. | 100 |  |  |
| 120 | Harness straps. Inspect pilot seat harnesses and passenger harnesses. Check the operation of the harness locks. | 100 | 24-10.01 |  |
| 121 | Onboard first aid kit. Inspect the first aid kit, check its completeness and expiration dates of the medicines it contains. | 100 | - |  |
|  | RESCUE EQUIPMENT |  |  |  |
| 122 | Emergency beacon ACK E-04 ELT. Inspect the emergency beacon and check its operation. | 100 |  | If provided (optional) |
| 123 | Parachute rescue system GRS 6 800-990 SDS 175 m2. Inspect the components of the ballistic parachute rescue system. | 100 | 25-70.03 RE | Instruction Manual for Assembly and Use paragraph 13_page 38 |
| 124 | Treat the steel parts of the rescue system with silicone grease | 100 |  |  |
| 125 | Rescue system hatch Inspect the "bouncer" hatch of the rescue system. | 100 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
|  | FIREFIGHTING EQUIPMENT |  |  |  |
| 126 | On-board fire extinguishers PYROCOOL GDP-400. Inspect on-board fire extinguishers and their mounting points Check the expiration date of fire extinguishers. | 100 |  |  |
|  | VENTILATION SYSTEM |  |  |  |
| 127 | Ventilation System. Inspect the cabin ventilation system. Check system operation with the engine running. | 100 | 21-00.01 |  |
|  | AIRCRAFT CONTROL |  |  |  |
| 128 | Aileron control system Inspect the aileron control system and check its operation. | 100 |  |  |
| 129 | Aileron deflection angles Measure aileron deflection angles. | 600 |  |  |
| 130 | Directional rudder control system Inspect the rudder control system and check its operation. | 100 |  |  |
| 131 | Directional rudder deflection angles Measure rudder deflection angles. | 600 |  |  |
| 132 | Altitude rudder control system Inspect the rudder control system and check its operation. | 100 |  |  |
| 133 | Altitude rudder deflection angles. Measure rudder deflection angles. | 600 |  |  |
| 134 | Altitude rudder trimmer control system Inspect the rudder trimmer control system and check its operation. | 100 |  |  |
| 135 | Angles of deflection of the elevation trimmer. Measure rudder trimmer deflection angles. | 600 |  |  |
| 136 | Flap control system. Inspect the flap control system. Perform control release and retraction of the flaps. | 100 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
| 137 | Flap deflection angles. Measure flap deflection angles. | 600 |  |  |
| 138 | Linkage Nodes. Dismantle: ailerons, flaps, rudder, rudder trimmer, rudder direction. Perform defect inspection of linkage assemblies, axles and mounting bolts. Install ailerons, flaps, rudder, elevator, rudder trimmer, and rudder. | 600 |  |  |
|  | HYDRAULIC SYSTEM |  |  |  |
| 139 | Inspection of the hydraulic system. Inspect hydraulic system units and lines. | 100 | 29-00.4.1 |  |
| 140 | Refilling with hydraulic fluid. Check quantity of hydraulic fluid in tank, refill (if necessary). | 100 | 12-10.2.11 |  |
| 141 | Changing the hydraulic fluid Replace the hydraulic fluid. | 5 years | 12-10.2.12 |  |
| 142 | Checking the function of the hydraulic system. Carry out a control retraction and release of the landing gear. | 100 | 32-30.4 |  |
| 143 | Check emergency landing gear release. Perform a control emergency landing gear release. | 100 |  |  |
| 144 | Tightness of the hydraulic system Check the hydraulic system for leaks. | 100 | 29-00.4.2 |  |
|  | CHASSI |  |  |  |
| 145 | Place the airplane on the elevators. | - | 07-00.01 |  |
| 146 | Main landing gear supports Inspect the main landing gear supports. | 100 |  |  |
| 147 | Wheels of the main landing gear supports. Dismantle, inspect and install the wheels of the main undercarriage supports. | 100 |  |  |
| 148 | Front chassis support Inspect the front landing gear support. | 100 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
| 149 | Front chassis support wheel. Dismantle, inspect, mount the front landing gear support wheel. | 100 |  |  |
| 150 | Pressure in the wheel pneumatics Check pressure in pneumatics of front and main landing gear wheels. | 100 | 12-20.01 |  |
| 151 | Front chassis support pivot control system. Inspect and check the operation of the front support pivot control system. | 100 |  |  |
| 152 | Angles of rotation of the front chassis support. Measure the angles of rotation of the front support. | 600 |  |  |
| 153 | Take the airplane off the elevators. | - | 07-00.01 |  |
|  | BRAKE SYSTEM |  |  |  |
| 154 | Inspection of the brake system Inspect the brake system units and lines | 100 |  |  |
| 155 | Fill with brake fluid Refill (topping up) the system with brake fluid. | 100 | 12-10.04 | If necessary |
| 156 | Changing the brake fluid Change the brake fluid in the system. | 5 years | 12-10.04 |  |
| 157 | Brake inspection. Check that the brakes are working properly. | 100 |  |  |
| 158 | Tightness of the brake system Check the brake system for leaks. | 100 |  |  |
|  | FUEL SYSTEM |  |  |  |
| 159 | Fuel system Inspect fuel system units and lines. | 100 |  |  |
| 160 | Fuel tanks. Inspect tanks from inside, clean if necessary, check for leaks. | 600 | 28-10.01 |  |
| 161 | Fuel gauge sensor. Check operation and correctness of fuel gauge sensor readings. | 600 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
| 162 | Fuel pumps Magneti Marelli MAM00026M Check the fuel pumps for proper operation. | 100 |  |  |
| 163 | # Alpha Filte AF 454 fuel purification filter Perform filter replacement. | 100 |  |  |
| 164 | Tightness of the fuel system Inspect the fuel system and pressure test it for leaks. | 100 |  |  |
| 165 | Sufflation and drainage system. Inspect and blow out fuel tank drain tubes with compressed air. | 100 |  |  |
| 166 | Checking the fuel system flow capacity. Perform a fuel system flow capacity check. | 600 |  |  |
|  | POWER PLANT |  |  |  |
| 167 | Engine Installation. Inspect the engine frame and engine mounting components. | 100 |  |  |
| 168 | ROTAX 918 EP engine. Perform engine maintenance according to the hours of operation. |  | Service Manual ROTAX 918 EP (AE_MML_918 EP) |  |
| 169 | Air Supply System. Inspect the air receiver and air filter box. | 100 |  |  |
| 170 | Air Filter WS-002TV PS-0236. Disassemble, wash, inspect, impregnate, and install the air filter. | 100 | Service Manual ROTAX 912_12-20-00_p.2 |  |
| 171 | Intercooler box flap. Inspect and check the operation of the intercooler box damper. | 100 |  |  |
| 172 | Liquid cooling system. Inspect the lines, radiator and thermostat of cooling system. | 100 |  |  |
| 173 | Changing the coolant Change the coolant. | 5 years |  |  |
| 174 | Lubrication system. Inspect the lines, oil tank, radiator, and thermostat of lubrication system. | 100 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
| 175 | Oil tank. Flush the oil tank (if necessary and/or if leaded fuel is used). | 200 | Service Manual ROTAX 918 EP_12-20-00 |  |
| 176 | Engine Management Inspect and check the operation of the engine management system. Check throttle force adjustments. | 100 |  |  |
| 177 | Exhaust system Inspect the exhaust system. | 100 |  |  |
|  | AIR PROPELLER |  |  |  |
| 178 | Propeller shoe Dismantle the propeller bark, clean it from dirt, inspect it, reassemble the bark. | 100 | RES RE VES ANG18R |  |
| 179 | Propeller blades. Dismantle the blades, inspect the blades and propeller hub, and install the blades. | 100 | RES RE VES ANG18LF |  |
| 180 | Air propeller control system. Inspect the propeller control system, check its operation. | 100 | 61-20.3.1 RE |  |
|  | AUTOPILOT |  |  |  |
| 181 | Autopilot. Inspect the Dynon SV-32 and Dynon SV-42 autopilot servos. Perform autopilot calibration and test | 100 | DYNON AVIONICS SkyView System Installation Guide |  |
|  | COMMUNICATIONS EQUIPMENT |  |  |  |
| 182 | Radio station (SV-COM-X83 module with SV-COM-T8 unit). Inspect the components of the radio, check its operation. | 100 | 23-10 | DYNON AVIONICS SkyView System Installation Guide |
| 183 | INTERCOM 403MC airplane intercom system. Inspect the NTERCOM 403MC ATP, aviation headsets, and check their operation. | 100 | 23-50.01 | INTERCOM 403MC Installation and Users Guide |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
| 184 | Connections, harnesses, wires, connectors Inspect antenna feeders, connections, harnesses, wires, connectors, electrical power supply to communications equipment. | 100 |  |  |
|  | ELECTRICAL EQUIPMENT |  |  |  |
| 185 | Connections, harnesses, wires, connectors, electrical power Inspect connections, harnesses, wires, connectors, electrical power supply to electrical equipment on the engine and airframe. | 100 |  |  |
| 186 | Battery ALIANT YLP24 Check battery voltage. Remove and inspect the battery, its place of installation. Check condition of wires and contacts, clean (if necessary) battery surfaces and wires. Install and secure the battery. | 100 | 24-30.01 | ALIANT CB12 (charger) |
| 187 | SV-BAT-320 rechargeable battery Remove, inspect, perform a test check of the battery pack. Install and secure the battery. | 100 | 24-30.02 | DYNON AVIONICS SkyView System Installation Guide |
| 188 | Integrated generator. Check the function of the integrated engine generator. | 100 |  |  |
|  | LIGHTING EQUIPMENT |  |  |  |
| 189 | DURALUX-660 landing lights. Inspect the landing lights, check their operation. | 100 |  |  |
| 190 | Onboard lights, strobe lights. Inspect the airborne aeronautical lights and strobe lights and check their operation. | 100 |  |  |
|  | PILOTING, NAVIGATION, INSTRUMENTATION EQUIPMENT |  |  |  |
| 191 | Static and dynamic pressure systems Inspect elements of static and dynamic pressure systems. | 100 |  |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
| 192 | Static and dynamic pressure systems Disconnect the static and dynamic pressure lines and blow them out with compressed air. Connect the trunk lines. | 200 |  |  |
| 193 | Dynon Heated AOA/Pitot Probe Air Pressure Receiver. Inspect the Pitot tube, check the operation of the LDPE heating system. | 100 |  |  |
| 194 | Moisture filters. Inspect moisture filters, static and dynamic pressure system piping. | 100 |  |  |
| 195 | Operability of full and static pressure systems. Inspect and check the tightness and operability of the full and static pressure systems using a test device. Check for compliance with specifications: - Vertical speed indicator (variometer) Vertical speed BC-6A; - ASI 180M-3 airspeed indicator; - Altimeter ALTN6ME-2; - SV-ADAHRS-200 module sensor. | 100 |  |  |
| 196 | Stall speed sensor. Check the stall speed sensor for proper operation. | 100 | DYNON AVIONICS SkyView System Installation Guide |  |
| 197 | INCLINOMETER BI2 slip indicator. Inspect the slip indicator, check its function. | 100 |  |  |
| 198 | SKYSPORTS PEDESTAL MOUNT COMPASSES Magnetic Compass. Inspect the magnetic compass and check that it is working properly. | 100 |  |  |
| 199 | Dynon SV-HDX1100 complex. Inspect the Dynon Skyview 1000 and check that it is working properly. | 100 | DYNON AVIONICS SkyView System Installation Guide |  |

| item RO | Name of service object and content of works | Period | TC | Note |
| --- | --- | --- | --- | --- |
|  | FINAL WORKS |  |  |  |
| 200 | Make sure that in the cockpit "MASTER SWITCH" and "IGN A" "IGN B" (ignition circuit switches) are in the "OFF" position, the safety check of the ballistic parachute system is set. | - | - |  |
| 201 | Inspect the airplane inside, install engine cowls, hatches, tail cowl, removable inner and outer hatches. | - |  |  |
| 202 | Perform an external inspection of the airplane en-route. | - |  |  |
| 203 | Perform PARKING (P) to ensure the aircraft is parked. |  |  |  |

10. STORAGE MAINTENANCE

10.1 Aircraft and engine preservation works for up to 1 year (see TC 10-30.01)

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
|  | PRELIMINARY WORKS |  |  |
| 204 | Take the covers off the airplane. |  |  |
| 205 | Remove water, snow, ice, frost from the aircraft surface, if any. | 12-30.01 |  |
| 206 | Unlock cabin and trunk door locks and open them. |  |  |
| 207 | Remove the mooring tackle. |  |  |
| 208 | Disconnect and remove the ground wire. |  |  |
| 209 | Unlock rudder and ailerons. |  |  |
|  | AIRCRAFT PRESERVATION |  |  |
| 210 | Remove the engine cowls. |  |  |
| 211 | Perform work on the airplane to the extent of Preliminary Training (DY). |  |  |
| 212 | Perform full refueling (refueling) of the aircraft with fuel and operating fluids. | 12-10.01 |  |
| 213 | Remove the battery pack and put it away for charging and storage. |  |  |
| 214 | Remove and deposit aviation headsets. |  |  |
| 215 | Clean and wash the airframe (inside and outside), glazing, landing gear, engine, propeller, oil and water radiator honeycomb. | Operators Manual_ROTAX 918 EP_ Operators Manual_ROTAX 918 EP_ 12-20-00- page 3 |  |
| 216 | Apply TsIATIM-201 grease to metal parts and surfaces of the airplane that do not have a protective coating. |  |  |

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
|  | ENGINE PRESERVATION |  |  |
| 218 | Perform work on the engine to the extent of the Preliminary Preparation (DY). | ROTAX-918 EP Engine Operation Manual (Operators Manual For Rotax® Engine Type 918 EP) п. 8.1 Installation Manual for ROTAX-918 EP motors п. 10.10 |  |
| 219 | Warm up the engine for 5 min at stabilized temperatures (oil and coolant temperature should be between 50 and 70°C). Turn off the engine. |  |  |
| 220 | Let the engine cool down. |  |  |
| 221 | Perform an oil change. |  |  |
| 222 | Remove the upper spark plugs and pour anti-corrosion oil into each cylinder through the plug holes. |  |  |
| 223 | Manually turn the air screw several times to ensure that the corrosion protection oil reaches all necessary points and surfaces of the engine. |  |  |
| 224 | Screw in the upper spark plugs. |  |  |
| 225 | On a cold engine, install plugs on all intake, exhaust, and drain ports, including the exhaust system outlet. |  |  |
| 226 | Apply anti-corrosion oil to the outer surfaces of the engine. |  |  |
|  | FINAL WORKS |  |  |
| 227 | Install the hoods. |  |  |
| 228 | Perform PARKING (P) to ensure the aircraft is parked. |  |  |

10.2 Work on the airplane and engine after 3 months of storage (see TC 10-30.02)

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
| 229 | "Disarm the airplane." |  |  |
| 230 | Remove the covers from the airplane and dry. | 10-10.02 |  |
| 231 | Check the sealing of the airplane. |  |  |
| 232 | Remove dust, water, snow, ice, frost from the aircraft surface (if necessary). | 12-30.01 |  |
| 233 | Unlock the cabin and luggage compartment door locks and open them Open the hatches. Ventilate the airplane |  |  |
| 234 | Turn the wheels of the undercarriage supports to 120o. |  |  |
| 235 | Check that the drainage holes on the airplane are clean. |  |  |
| 236 | Perform an external inspection of the aircraft en route, inspect the aircraft inside, make sure that there are no corrosion damages and paint coating violations, fuel, oil, coolant, hydraulic and brake fluid leaks. |  |  |
| 237 | Hoods off. Perform external inspection of engine, make sure there is no corrosion damage, oil, coolant, hydraulic fluid leakage. |  |  |
| 238 | Install the battery, check the operability of the radio equipment within the scope of preflight maintenance. |  | Do not start the engine |
| 239 | Install engine cowls. Close hatches, close and lock cab and luggage compartment doors. |  |  |
| 240 | Perform PARKING (P) to ensure the aircraft is parked. |  |  |

10.3 Aircraft and engine de-conditioning work (see TC 10-30.3)

| item RO | Name of service object and content of works | TC |  | Note |
| --- | --- | --- | --- | --- |
|  | PRELIMINARY WORKS |  |  |  |
| 241 | "Disarm the airplane." |  |  |  |
| 242 | Remove the covers from the airplane and dry. | 10-10.02 |  |  |
| 243 | Check the sealing of the airplane. | 6-10 ROS. |  |  |
| 244 | Remove dust, water, snow, ice, frost from the aircraft surface (if necessary). | 12-30.01 |  |  |
| 245 | Unlock the cabin and luggage compartment door locks and open them Open the hatch covers. Air the airplane. |  |  |  |
|  | AIRCRAFT DE-CONDITIONING |  |  |  |
| 246 | Check that the drainage holes on the airplane are clean. |  |  |  |
| 247 | Remove external preservation lubricant from metal parts and surfaces of the airplane that do not have a protective coating. |  |  |  |
| 248 | Perform an external inspection of the airplane en route, inspect the airplane inside, make sure that there are no corrosion damages and paint coating violations, fuel, oil, coolant, hydraulic and (or) brake fluid leaks. |  |  |  |
| 249 | Install the battery, check its technical condition. |  |  |  |
| 250 | Pick up from warehouse and connect aviation headsets. |  |  |  |
| 251 | Check the technical condition of the SV-BAT-320 backup battery pack |  |  |  |
| 252 | Remove the engine hood covers. |  |  |  |
| 253 | Perform work on the airplane to the extent of Preliminary Preparation - DAILY CHECK (DY). |  |  |  |
|  | PROPELLER DISASSEMBLY |  |  |  |
| 254 | Carry out work on deconservation of the propeller. | RES RE VES ANG18LF |  |  |
|  | ENGINE PRESERVATION |  |  |  |
| 255 | Remove all plugs and clean the engine of external preservative. | Engine manual ROTAX-918 EP (Operators Manual For Rotax® Engine Type 918 EP) п. 8.2 |  |  |
| 256 | Remove and clean the spark plugs. Install the spark plugs in their places |  |  |  |

| 257 | Perform work on the engine in the scope of Preliminary Preparation - DAILY CHECK (DY). |  |  |
| --- | --- | --- | --- |
|  | FINAL WORKS |  |  |
| 258 | Install the hoods. |  |  |
| 258А | Perform PARKING (P) to ensure the aircraft is parked. |  |  |
| 258В | Perform a control flight (overflight) of the aircraft (if the storage period was more than 6 months). |  |  |

# 11. SPECIAL TO.

11.1 Work performed after an aircraft enters areas of intense turbulence and thunderstorms

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
| 259 | Perform airplane maintenance to the extent of form 100 hours (inspection work only). |  |  |
| 260 | Check the operation of electrical and radio equipment without removing it from the airplane. |  |  |
| 261 | Check static and dynamic piping for leaks. |  |  |

11.2 Work performed after an aircraft flight in excess of operational overloads

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
| 262 | Perform airplane maintenance to the extent of form 100 hours (inspection work only). |  |  |

11.3 Work performed after a rough landing of the aircraft

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
| 263 | Perform airplane maintenance to the extent of form 100 hours (inspection work only). |  |  |
| 264 | Check the leveling of the aircraft. |  | If necessary |

11.4 Work performed after an aircraft has entered storm conditions on the ground

| item RO | Name of service object and content of works | TC | Note |
| --- | --- | --- | --- |
| 265 | Perform airplane maintenance to the extent of form 100 hours (inspection work only). |  |  |

12. LUBRICATION SCHEDULE

12.1 In order to ensure flight safety, reduce wear and friction, and extend the service life of moving  rubbing joints, lubricate them within the time limits specified in the lubrication cards.

Lubrication of parts and assemblies is performed during periodic maintenance or within the period specified in lubrication cards.

12.2 Use only clean grease.

Before applying grease, clean old grease and contaminants from the joint or assembly. Take measures to prevent contaminants (sand, dust, water, etc.) from entering the lubricated parts and assemblies together with the lubricant

After lubricating the joint or assembly, remove excess grease from its surface.

12.3 When carrying out lubrication work, use clean cloths, clean hair brushes and syringes.

Lubrication of linkage hinges should be performed without disassembly of joints by treating them with aerosol preparation WD-40 or its analogues by pouring them into joint gaps.

If the parts to be lubricated were removed from the airplane (removal of defects, modifications, etc.), then when installing them in place, lubricate the joints with the grease specified in the "ANG-01 Airplane Operation Manual".

12.4 Lubrication of assemblies of finished products should be performed in accordance with the operating instructions or data sheets for these products.

12.5 When performing lubrication work, apply the information in the operating manual on lubrication points, timing of such work, equipment and grease grades used for this purpose.

13. LUBRICATION CARDS

| item RO | Lubrication points | Work content | Applicable lubrication | Frequency of performance |
| --- | --- | --- | --- | --- |
|  | PARACHUTE RESCUE SYSTEM |  |  |  |
| 266 | Rescue system activation axle and handle | Apply aerosol lubricant | WD-40 (or its equivalent) | after 100 h flying time |
| 267 | Steel parts of the rescue system | ## Apply silicone spray to clean surface of steel parts | Repsol Moto Silicone Spray (or its equivalent) | after 100 h flying time |
|  | POWER PLANT |  |  |  |
| 268 | Exhaust system swivel joints | Rinse with gasoline B-70, wipe it off with a clean cloth, lubricate | LOCTITE Anti-Seize 8151 | after 100 h flying time |

14. APPENDICES.

Appendix 1:     Technical parameters of aircraft and engine systems controlled during maintenance work

| item RO | Technical parameters | Indicators | Note |
| --- | --- | --- | --- |
|  | POWER PLANT |  |  |
| 269 | Amount of oil in the oil tank | 3 л | Checking by oil tank gauge |
| 270 | Total amount of fuel in tanks | 275 л | Fuel gauge control |
| 271 | Quantity of fuel in each wing caisson tank | 110 л |  |
| 272 | Quantity of fuel in the consumable caisson tank | 55 л |  |
|  | CHASSI |  |  |
| 273 | Turning the front wheel to the left and right from the neutral position | +30° | At full pedal travel |
| 274 | Maximum air pressure in pneumatics of wheels of chassis support wheels | 3+ 0.3 bar |  |
| 275 | Initial gas pressure in chassis strut shock absorbers | 20 bars |  |
| 276 | Full stroke of chassis strut shock absorber rods | 80 mm |  |
|  | MANAGEMENT |  |  |
| 277 | Altitude rudder deflection: - downward (RTC travel from neutral position 16° "away") - upwards (RTC travel from neutral position 15° "towards yourself") | 19° 29° |  |
| 278 | Aileron deflection angles | 25° up +2° 18° down +2° | When the RTC is fully tilted to the left or right of the neutral position |
| 279 | Rudder deflection angles | +29° | With full pedal travel from neutral position + 16° |
| 280 | Flap deflection angles | 10° ; 20° ; 30° | Control by pointer in the cab |
| 281 | Aileron trimmer plate deflection | +10° | Maximum permissible bend |
| 282 | Deflection of the rudder trimmer plate | +10° | Maximum permissible bend |
| 283 | Altitude rudder trimmer deflection | +15 1°+° |  |

| item RO | Technical parameters | Indicators | Note |
| --- | --- | --- | --- |
|  | HYDRAULIC SYSTEM |  |  |
| 284 | Pressure in the hydraulic system (chassis retraction and release system) | 50 bar | Pressure control by pointers in the cockpit |
| 286 | Hydraulic tank capacity | 2,5 л |  |

Appendix 2.     List of operations after which a check flight (overflight) of the airplane is performed

| item RO | Name of works | Forms of maintenance performed after control flight (overflight) |
| --- | --- | --- |
| 287 | Engine change | Form 100 hrs. |
| 288 | Air propeller replacement | DAILY CHECK (DY) |
| 289 | Replacement of power components and controls: - of the wing; - of the rudder; - of the rudder; - aileron; - flap; - front or main landing gear support. | DAILY CHECK (DY) |
| 290 | After the aircraft has been stored for more than 6 months. | DAILY CHECK (DY) |

Appendix 3.     Recommended operating fluids for use on the airplane

| item RO | Name | Mark | Note |
| --- | --- | --- | --- |
| 291 | Motor gasoline | AI-95; A92; AKI91; MOGAS EN 228 EN 228 super, EN 228 super | Operating Instructions ROTAX 918 EP_p.2.3) See service manual SI-918 EP i-001 |
|  | Aviation gasoline | AVGAS 100 LL (ASTM D910) |  |
| 292 | Motor oil | Aeroshell Sport Plus 4 (according to RON 424 classification) | Operating Instructions ROTAX 918 EP_p.2.4) See service manual SI-918 EP i-001 |
| 293 | Coolant | HEPUG (Shell Dex-Cool is recommended | Operating Instructions ROTAX 918 EP_p.2.2) See service manual SI-918 EP i-001 |
| 294 | Brake fluid | DOT 5 Silicone Brake Fluid is a silicone based fluid that meet Mil PRF-46176 & 46176B | DOT 5 silicone brake fluid that meets Mil PRF-46176 and 46176B requirements. |
| 295 | Hydraulic fluid | ATF DEXRON IV | Shelf life - in closed containers - 5 years; |